### Flip Top Table Operation Guide

Rack Flip Top Table under-frames are designed and constructed to be easily operated by a single user enabling speedy re-configuration of a room or workspace.

Where the frames are stacked in the “Open” position and are to be set up ready for use, they are simply operated as follows:

- Release the braked castors on the front feet.

- Wheel the frame into the desired position and re-lock the braked castors.

- Simply place your knee gently against the cross rail of the frame and pull the very top edge of the worktop towards yourself until the worktop reaches the “Closed” position, where you will hear an audible click and feel the worktop locate firmly in this position.

It should be noted that the frames do not lock in the “Open” Position.

In order to re-configure or stack the frames away, the operation is as follows:

- Determine the side of the frame where the pull release bar is located.

- Place the heels of your hands a good distance apart against the leading edge with your fingers under the worktop, locate the release bar with your fingers and simply pull and hold, there will be an audible click as the mechanism releases.

- Whilst holding the release bar, lift the top by approximately 50mm from the “Closed” position, the release bar can then be let go, and the worktop returned entirely to the “Open” position.

- Release the braked castors and move the frame to the desired position.

- Re-lock the castors.

It should also be noted that if the action of pulling the release bar is firm, this action can be eased by “wiggling” the worktop slightly as pressure is applied to the pull bar.